volatile in C — Notes

What is volatile?

- A type qualifier in C (like const)

- Tells compiler: "always read/write this variable from actual memory(ram)"

- Prevents compiler optimization on that variable

Why needed?

- Compiler normally caches variables in CPU registers for speed

- But some variables change outside the compiler's knowledge:

  - Hardware registers

  - Interrupt Service Routines (ISR)

  - Another thread / signal handler

Syntax

c

volatile int flag;                    // simple variable

volatile int *ptr;                    // pointer to volatile int

const volatile int *reg = 0x4000;     // read-only hardware register

Memory Sections — Where does it live?

┌──────────────┐

│    Stack               │ ← volatile local vars   (rare)

├──────────────┤

│     Heap              │ ← volatile via pointer  (malloc + cast)

├──────────────┤

│     Data               │ ← volatile globals/static  ✅ most common

├──────────────┤

│     Code              │ ← never volatile  ❌

└──────────────┘

volatile is a qualifier, not a memory region — it can sit in any section

Without vs With volatile

|  | Without | With |
| --- | --- | --- |
| Compiler behavior | Caches in register | Re-reads from RAM |
| Assembly reads | Once | Every access |
| Risk | Stale value / infinite loop | Correct behavior |